Best Practices for D-Bus-2 User

A guide to avoid most common development mistakes

# Messages may disappear or be repeated

First thing you have to keep in mind about the D-Bus frames is that under very rare circumstances the D-Bus-2 messages may be ignored or send multiple times (e.g. Acknowledge is corrupted in such way that only one controller doesn’t see it correctly). Please develop your programs with that fact in mind.

# Avoid using Broadcasts for mission-critical functions

Related to section Messages may disappear or be repeated.

Broadcast is a convenient way to distribute the message to many counterparts. It is, however, a very unsafe way to do so. As the Acknowledge is being transmitted by the sender during the broadcast, certain errors on the D-Line may lead to the recipients getting wrong data (and therefore discarding the message) without the sender noticing.

Only consider using broadcast if a message may be skipped safely.

# Don’t trust Acknowledge for mission-critical functions, use Request-Reply schema instead

Related to section Messages may disappear or be repeated.

For the mission-critical functions you should also avoid relying on Acknowledge for confirmation even with normal messages. ACK_OK only means that the controller has received the message and that that message was a correct D-Bus-2 frame.

ACK_OK is generated for any message that is a valid frame, even if the message does not exist in any of the Receive Tables. It does not mean that the controller has actually done something about the message.

Request-Reply schema is more robust and should be employed for the things that are mission-critical and/or potentially dangerous.

# Avoid toggle messages, use separate messages for on/off instead

Related to section Messages may disappear or be repeated.

As a message can (in theory) be lost, it is not recommended to use exactly the same message to toggle something, especially with mission-critical and/or dangerous functions. Either use different sets of data or different messages for on and off.

# Keep the bus load below 70%

D-Bus is a very robust protocol with an ability to recover from most failures. However, heavy load on the D-Line might cause unexpected consequences such as delays in the user interface or other unexpected behaviour.

Therefore, idle times are needed in order for every controller to be able to send D-Bus messages, especially in heterogeneous systems for controllers with a slightly lower clock speed.

# Keep the messages short

Related to section Keep the bus load below 70%.

D-Bus is a command bus, not a multimedia bus. Hence, the messages have to be kept short. Otherwise, the bus may be blocked for too long.

For example, a message with the length of 255 @9600 baud will block the D-Line for about 0,33 seconds.

# Event trigger instead of polling

Related to sections Keep the bus load below 70%.

To reduce the potential bus load, it is recommended to use event triggering instead of periodic polling.

# BIT_TIME must be set according to used baudrate

When setting up the UART, the BIT_TIME define must be set in relevance to the baudrate. The BIT_TIME is capped at 38400 and stays at 26 microseconds for higher baudrates.

| Baudrate | Bit Time (Microseconds) |
| --- | --- |
| 9600 | 104 |
| 19200 | 52 |
| 38400 | 26 |
| 76800 | 26 |
| 125000 | 26 |
| 250000 | 26 |